Purpose:

The purpose of this examination of a 9V battery is to determine the voltaic power and the inner resistance in the battery.

Theory:

battery

consumer with

resistance Rload

A power source can be described in the following way:

The source consist of a perfect voltage source, which is able to keep the same potential difference at all times. This source is connected to an inner resistor (load), in which energy is deposited (i.e lost). This is shown in the dotted box in the diagram to the right.

The circuit also shows an outer resistor - a load, Rload - where the energy for som other purpose is deposited (a consumer).

The potential difference of the perfect voltage source is called the circuit voltage U0 and the potential difference across the outer load is called the terminal voltage Uterm

Not all powersources follow this model but if they do, it’s possible to determine the circuit voltage and the inner resistance Ri in the following way:

The total drop in potential through the entire circuit is equal to the circiut voltage U0. This potential difference can be divided into one part across the inner restistor and an other part across the outer load, that is

U0=Ui+Uterm

The potential difference across the outer load would be the terminal voltage Uload and, using Ohm’s Law, we can describe the potential difference across the inner resistor Ri  by  Ui=I·Ri.

This gives us

U0=Ri·I+Uterm

By changing the resistance of the load Rload we can change the current in the circuit and measurring the terminal voltage as a function of the current in the circuit we can plot a graph of (I, Uterm)

Using the mathematical model of a straight line (in danish terms)

y=ax+b

we can determine the value of the inner resistor and the curcuit voltage doing regression and compare it to the psycical theory

Uterm=U0-Ri·I

How to do it:

DO NOT CONNECT THE BATTERY UNTIL YOUR SET-UP HAS BEEN APPROVED

- Build a circuit as shown on the whiteboard. As outer resistor (load) we will use a long, thin wire. The resistance is altered by changing the length of the wire.

- Use

  - Capstone sensor (current- and voltage-) for meassuring the terminal voltage Uterm using manual data collection.

  - Amp-meter / Multimeter for measurring current

at different loads (values of outer resistor).

- Make at least ten meassurements starting with small load (long wire) and increase the load by meassuring on an increasingly shorter wire.

- Disconnect the circuit when not meassuring in order not to drain the battery

Using the data

- Make a graph showing the terminal voltage as a function of the current - a (I, Uterm) - plot.

- Determine the values of the circuit voltage U0, the inner load Ri and the maximal current Imaks the battery can supply.

- Calculate the power output from the battery using

P=Uterm·I

- Calculate the outer load using Ohm’s Law

Rload=UtermI

- Make a graph showing the power as a function of the outer resistance Rload, - that is plot Rload on the x-axis and P on the y-axis. Compare the value of the outer resistor Rload to the value of the inner load Ri that you found earlier.